Safe Flame: Behavior Aware LPG Safety and Consumption Prediction System

Introduction

Some similar work exists upon literature review in 2025 and 2026.These systems are mostly implemented using IoT and AI. Several LPG monitoring and safety systems have been developed in recent years focusing primarily on gas leakage detection and alert mechanism. They typically use gas sensors interfaced with microcontroller, to detect the presence of LPG in the surrounding environment.

Common Features of Existing Systems

- Detection of LPG leakage using gas sensor.

- Activation of alarm on detection.

- Sending notification via GSM/IoT based platform.

- Basic gas level monitoring using load sensors.

- Automatic shut off valves in certain implementations

These systems are largely reactive in nature in case of hazardous condition.

Limitations of Existing System

- Reactive operation

- Lack of context awareness: -gas sensors detect the presence of LPG, but cannot distinguish between normal cooking usage, minor leakage and hazardous leakage. This leads to false alarm.

- Absence of behavioral analysis: -existing systems does not analyze user consumption patter, time-based usage trend and abnormal usage behavior.

- Limited prediction capability: -Only few systems estimate remaining quantity of gas.

- No leak classification: -Most designs cannot differentiate between,

- slow leak –gradual weight loss

- sudden leak –rapid weight loss

- normal usage-time bound usage pattern

Key Enhancement of Proposed System

- Behavior based analysis: -learns daily gas usage pattern and establish baseline for normal operation. Any deviation is treated as anomaly.

- Leakage classification mechanism.

- slow leak –gradual weight loss

- sudden leak –rapid weight loss

- normal usage-time bound usage pattern

- Predictive gas consumption: -using historical data, the system estimates:

- Remaining gas quantity

- Expected duration of availability

- Multi-parameter decision making- the system combines:

- Load cell data (weight change)

- Gas sensor output

- Time-based usage patterns

- Anomaly Detection: -abnormal behaviors such as:

- Gas usage at unusual hours.

- Continuous consumption beyond normal duration.

are identified and flagged.

Algorithm

- Weight change:

Current weight-previous weight

- Rate of change:

ΔW/Δt

- Classification

if Rate>upper threshold=>Sudden risk

if rate>lower threshold => Slow leak

else =>Normal

- Prediction

Remaining days=remaining gas/avg daily usage

- Behavior detection

If usage not in normal time=>Anomaly

Proposed system improves upon existing solution by shifting from reactive detection mode to predictive and behavior aware system by incorporating data analysis, pattern recognition and multi-parameter decision making. Thus, the system enhances safety, reliability, user awareness and preventive capability.

System Level Block Diagram

Components Required

- 50 kg load cell with HX711 amplifier

- MQ-2 -LPG sensor

- MQ-4 -Methane sensor

- MQ-7 -CO sensor

- DHT22 -temperature and humidity sensor

LPG leaks are dangerous, but methane and CO are equally hazardous. Methane is a major component of natural gas and CO is a byproduct of incomplete combustion. Detecting all three ensures protection not only from leaks but also from faulty burners or ventilation issues. In kitchen, industrial setup or vehicles LPG and methane can coexist. CO often builds up when LPG burns inefficiently – so monitoring CO helps identify poor combustion or blocked exhaust before they become lethal.

High LPG+ low CO Leak

Normal LPG + high CO Incomplete combustion

High methane + high CO Mixed gas hazard

MQ-2 alone is fine for simple leak detection, but for distinguishing LPG, methane and CO a multi sensor array is used.

GPIO pin mapping

| Peripheral | STM32 Pin | Connector | Function |
| --- | --- | --- | --- |
| MQ-2 Sensor | PC0 | CN9 | ADC input (channel 10) |
| MQ-4 Sensor | PC2 | CN10 | ADC input (channel 12) |
| MQ-7 Sensor | PA5 | CN7 | ADC input (channel 5) |
| Load cell (HX711) | PB0 (DOUT) | CN 10 | GPIO input |
|  | PB1(SCK) | CN 10 | GPIO output |
| DHT22 | PA0 | CN10 | GPIO input |

STM32 SAR ADC gives us a digital value as output. Typically, value is in between 0 to 4095 (12- bit ADC). Following steps are involved for calculating the PPM concentration of MQ2, MQ4 and MQ7.

- Convert ADC value to voltage

Vout= (ADC value/4095) * Vref

- Compute sensor resistance (Rs)

Rs=((Vref-Vout)/Vout) * Rl

- Find the ratio Rs/Ro

Ro is the sensor resistance in clean air also called the calibration constant

- Apply sensor curve

Each sensor has a log-to-log curve given in the datasheet which helps us to calculate ppm values. It is approximated as:

Log 10(ppm)=a log10(Rs/Ro) +b; a and b are constants from datasheet

Temperature and Humidity sensor-DHT22

The DHT22 Sensor (also called AM2302) is a digital sensor that measures temperature and relative Humidity and sends the readings to a microcontroller using a single-wire digital communication protocol. Inside the DHT22 there are mainly:

A) Humidity Sensing Element

It contains a moisture-holding substrate between two electrodes.

- Water vapor changes the electrical properties (capacitance).

- The internal IC measures this change.

- It converts it into humidity percentage (%RH).

B) Temperature Sensor

It uses an NTC thermistor.

NTC means:

- Temperature ↑ Resistance ↓

The internal chip measures this resistance and converts it into temperature.

C) Internal ADC + Controller

An internal microcontroller:

- Reads humidity sensor

- Reads thermistor

- Converts analog values into digital form

- Sends 40-bit data serially

Communication Interface for DHT22

Step 1: MCU sends START signal

The microcontroller pulls the data line LOW for about:

- >1 ms

Then releases it HIGH.

Step 2: DHT22 responds

Sensor replies:

- LOW for 80 µs

- HIGH for 80 µs

This means:"I am ready to transmit."

Step 3: Sensor sends 40 bits

The data format is:

| Bits | Meaning |
| --- | --- |
| 16 bits | Humidity |
| 16 bits | Temperature |
| 8 bits | Checksum |

How Bits Are Encoded

Each bit starts with:

- LOW for 50 µs

Then HIGH pulse duration decides whether it is 0 or 1.

| HIGH Duration | Bit |
| --- | --- |
| ~26–28 µs | 0 |
| ~70 µs | 1 |

So MCU measures pulse width to facilitate data transmission.

Step 4: Data Format

The 40 bits are grouped into 5 bytes. The sensor sends the data in following format:

- Byte 1: - 8-bit Humidity Integer

- Byte 2: -8-bit Humidity Decimal

- Byte 3: - 8-bit Temperature Integer

- Byte 4: - 8-bit Temperature Decimal

- Byte 5: - 8-bit Checksum

Step 5: Interpretation of temperature and humidity values

The actual formula for calculating temperature and humidity is shown below:

But we use some bit level operations to read data from DHT22 sensor and output in digital format.

- Humidity= ((Byte1 << 8) | Byte2) / 10.0

- Temperature= ((Byte3 << 8) | Byte4) / 10.0

- If the MSB of Byte3 is set → temperature is negative.

- Checksum = (Byte1 + Byte2 + Byte3 + Byte4) & 0xFF

- Note that checksum must equal Byte5, otherwise reading is invalid.

- The left shift operation is done because, the DHT22 sends humidity and temperature as 16-bit numbers split into two 8-bit bytes. Byte1 is the high byte, Byte2 is the low byte. To reconstruct the full 16-bit value, you shift the high byte left by 8 bits and OR it with the low byte.

- In other words, (Byte1 << 8) | Byte2 is how you combine two 8-bit pieces into one 16-bit number. The left shift moves the high byte into the upper half, and the OR adds the low byte into the lower half.

- In the checksum calculation 0xFF is a hexadecimal constant that represents the binary value, 0xFF = 11111111 (binary) = 255 (decimal).

- The sum of the first 4 bytes could be larger than 255 (since each byte can be up to 255).

- But the checksum is only 1 byte (8 bits).

- 0xFF masks the result so only the lowest 8 bits are kept.

Interfacing DHT22 with STM32

- Connect the sensor with STM32 board. There are only three pins Vcc, GND and data. Connect Vcc with 3.3V supply from CN8, GND from CN8 and data pin is connected to PA0 in CN10 using jumper wires.

- DWT counter is enabled in the main code to enable precision timing in the range of micro seconds.

- The DHT22 sensor does not have any LED indication to identify whether it is powered ON when connected to STM32.To ensure that the sensor module works fine, measure the voltage between Vcc and GND pin using a multimeter.

- In the .ioc file configure PA0 as gpio input with pull-up option enabled. This is because DHT22 sensor require a 4.7K pull up resistor between Vcc and data pin.

- When the bus is idle, it is in high state condition. This is because of the pull up resistor.

- The DHT22 uses a single-wire bus for communication.

- That means neither the MCU nor the sensor actively drives the line high. They only pull it low when sending bits.

- To get a valid “high” level, you need a resistor that ties the line up to VCC. Without it, the line floats and the MCU reads random or stuck values.

- STM32 has internal pull-ups (~30–50 kΩ). These are too weak for reliable fast transitions.

- The DHT22 requires a stronger pull-up (typically 4.7 kΩ to 10 kΩ) to ensure clean rising edges within the microsecond timing window.

- That’s why you add an external 4.7 kΩ resistor between PA0 and 3.3 V.

- Figure below shows the output reading of DHT22 sensor.

- Figure below shows the timing diagram of DHT22 communication:

This DHT22 driver implements the sensor’s single-wire protocol with precise timing control. The microsecond delay function uses the STM32’s DWT cycle counter to generate accurate delays, while DHT22_WaitForPin ensures transitions occur within a timeout so the MCU never hangs. To start communication, the MCU drives the data pin low for 20 ms, then releases it and switches the pin to input mode; the DHT22 responds with a specific low-high handshake. The main read loop then captures 40 bits: each bit begins with a low pulse, followed by a high pulse whose duration encodes 0 or 1. The code samples the line after ~40 µs to decide the bit value and assembles the result into five bytes. A checksum verifies data integrity, and the raw humidity and temperature values are extracted from the first four bytes, scaled by 10 to yield human-readable floats. In essence, the code handles clock-like timing generation, protocol framing, error detection, and conversion, turning the DHT22’s pulse-width encoded signal into stable temperature and humidity readings.

Interfacing HX711+load cell with STM32

A three-wire load cell with HX711 amplifier module is connected with STM32.The hardware connection is as follows:

HX711 DT: -GPIO input- STM32 PB0-CN10

HX711 SCK: -GPIO output-STM32 PB1-CN10

HX711 Vcc: -5V- STM32 CN8

HX711 GND: - GND- STM32 GND CN8

A load cell is basically a metal bar with strain gauges that change resistance when the bar bends under force. This bending produces a very tiny voltage difference (in the range of millivolts). Because the signal is so small, the HX711 chip amplifies it about 128 times and then converts it into a 24-bit digital number. The STM32 microcontroller reads this number by toggling a clock pin (logic levels: 0 V for LOW, 3.3 V for HIGH) and sampling the data pin. Each read gives a raw count value that represents how much the bar is bending.

The program first performs a tare: it measures the raw counts when the platform is empty and stores that baseline. This ensures that “no load” corresponds to zero. Next, it performs calibration: you place a known weight (say 5 kg), the program calculates how many counts correspond to that weight, and from this it derives a conversion factor (kg per count). After that, every new measurement follows the formula:

Weight (kg)=(Raw counts-Tare offset)×Calibration factor

This way, the microcontroller continuously reads the HX711 output, subtracts the tare offset, multiplies by the calibration factor, and prints the net weight to PuTTY. Even when no object is placed, small fluctuations appear because the ADC is so sensitive to electrical noise and temperature drift, but the tare and filtering logic clamp those values close to zero.

This HX711 driver works by manually implementing the chip’s custom synchronous protocol through GPIO toggling. At initialization, PB1 (SCK) is set low to establish a stable starting state. During a read, the STM32 first waits until HX711 signals data ready by pulling PB0 (DOUT) low, then generates 24 clock pulses on PB1; each rising edge causes HX711 to shift out one bit of its 24-bit ADC register, which the code assembles into value. After the 24th bit, the code issues one extra pulse (25th), which is not part of the data but re-selects the next conversion mode — in this case Channel A with Gain 128, the default configuration. Because HX711 outputs a 24-bit two’s complement signed value, the MCU must sign-extend it into 32 bits so negative readings are preserved correctly. The tare routine averages multiple samples to establish a baseline offset, while the weight function subtracts this offset from new raw readings and multiplies by a calibration factor to convert counts into kilograms. In essence, the code handles clock generation, protocol framing, signed data conversion, and calibration, turning the millivolt signal from the load cell into a stable, meaningful weight measurement.

Figure below shows the DOUT and SCK signals captured using logic analyser (24MHz,8channel)

Figure below shows the generated clock signal and bit shifting operation.

- SCK ON time-10us

- SCK OFF time-10us

- Duty cycle -50 %

- Clock Frequency- 1/ (20 us) =50kHz

- A total of 25 pulses (24 clock pulses and channel selecting signal one) is generated at 50 kHz were each pulse take 20 us each.

- Hence, total read time-25 x 20 us =500 us =0.5ms

- Figure below shows the real time capture of read sequence.

- Time gap between two burst is determined by HX711 conversion rate.

- HX711 internal oscillator frequency-11.059 MHz.

- Rate pin tied to 0 (Default state) implies output data is at 10 Hz and conversion time is 100ms.

- Rate pin tied to 1 (Default state) implies output data is at 80 Hz and conversion time is 12.5ms.

- Conversion time =1/Data rate

- By default, DOUT is held high, when data is ready DOUT is pulled low, followed by SCK generation. At the end of 24 th pulse, DOUT goes high for conversion process.

Sensor Specifications

- MQ2-LPG sensor

| Parameter | Values |
| --- | --- |
| Detectable gases | LPG,i-butane, propane, methane, alcohol, hydrogen, smoke |
| Vc, Circuit voltage | 5V±0.1 AC OR DC |
| VH,Heating voltage | 5V±0.1 AC OR DC |
| RL, Load resistance can adjust | 20K Ω (typical) or (5K Ω - 47K Ω) |
| RH, Heater resistance 33Ω}5% | 33Ω ±5% |
| PH, Heating consumption | < 800mw |
| Rs, Sensing Resistance | 3KΩ-30KΩ |
| Rs/R0 | 9.8 |
| Interface type | ADC |

- MQ4-Methane sensor

| Parameter | Values |
| --- | --- |
| Detectable gases | Methane, natural gas, LNG |
| Vc, Circuit voltage | 5V±0.1 AC OR DC |
| VH,Heating voltage | 5V±0.1 AC OR DC |
| RL, Load resistance can adjust | 20K Ω (typical) |
| RH, Heater resistance 33Ω}5% | 33Ω ±5% |
| PH, Heating consumption | < 750mw |
| Rs, Sensing Resistance | 10KΩ-60KΩ |
| Rs/R0 | 4.4 |
| Interface type | ADC |

- MQ7- Carbon Monoxide sensor

| Parameter | Values |
| --- | --- |
| Detectable gases | Methane, natural gas,LNG |
| Vc, Circuit voltage | 5V±0.1 AC OR DC |
| VH (H), Heating voltage high | 5V±0.1 AC OR DC |
| VH (L), Heating voltage low | 1.4V±0.1 AC OR DC |
| RL, Load resistance can adjust | 20K Ω (typical) |
| RH, Heater resistance 33Ω}5% | 33Ω ±5% |
| TH (H) Heating time (high) | 60 ±1 seconds |
| TH (L) Heating time (low) | 90±1 seconds |
| PH, Heating consumption | < 350mw |
| Rs, Sensing Resistance | 2KΩ-20KΩ |
| Rs/R0 | 1.5 |
| Interface type | ADC |

- DHT22-Temperature and Humidity Sensor

| Parameter | Values |
| --- | --- |
| Power supply | 3.3 to 6V |
| Output signal type | Analog |
| Operating range | 0-100 % RH; temperature- -40 to 80o C |
| Resolution or sensitivity | Humidity 0.1% RH; Temperature 0.1o C |
| Interface type | Single bus protocol |

- Load cell +HX711

| Parameter | Values |
| --- | --- |
| Power supply | 2.6V to 5.5V (5V typical) |
| Output data coding | 2’s compliment |
| ADC resolution | 24 bits |
| No. of Differential input channels | 2-with selectable PGA gain of 32,64,128 |
| Interface type | Serial interface using GPIO |
| Output data rate | 10 Hz |
| Conversion time | 100 ms |
| Load cell sensitivity | 1 mV/V |

STM32 flat firmware code is updated to RTOS implementation. Five tasks have been created namely:

- Sensor Task- mq2, mq4, mq7, dht22, hx711 sensors.

- Process Task – core logic of the project such as weight monitoring, leak prediction etc.

- Control Task – In case of gas leakage or any other anomaly the auto cut of valve closes and buzzer sounds alarm.

- Communication Task-All the output values from different sensors displayed through UART.

- Compensation Task- mq2, mq4 and mq7 gas sensor readings may vary with accumulating temperature and humidity. Applying small compensation to the sensor curve reduce the chance of false alarm and trigger.

- Inter Process Communication Mechanism Queue was implemented to facilitate communication between the task.

- Two queues are named sensor queue and status queue respectively. Sensor queue is used to push the data to the processing task. The raw values from the ADC are packed into the structure sensor data which is passed to the process task for PPM calculation and other mathematical calculation.

- The status queue communicates between processing task and other tasks like control task, communication task and compensation task.

- A python code was generated to plot the gas concentration values of MQ2, MQ4 and MQ7.Figure below shows the output numerical values and graphical plot of sensor PPM output (time vs PPM).

- Load cell was interfaced in the following manner.

- The white wire of two adjacent load cells is tied together. Likewise, the black wire of adjacent load cells is tied together. Again, the red wire of diagonally opposite load cells is connected as E+, E-, A+ and A-. The positive and negative ends of the supply are connected with E+ and E- respectively.

- DT, SCK are connected to PB0 and PB1 followed +5V Vcc and GND.